1)

ADD X9 ,X0 ,X1

10001011000 00001 000000 00000 01001

X0, 00000,  will enter register file as “Read register 1” as it is Rn  (first register)

X1, 00001,  will enter register file as “Read register 2” as it is Rm (second register)

x9 , 01001, will enter register file as the “Write register” i.e. in which register data should be written to after ALU processing

ADD X10 ,X2 ,X3

10001011000 00011 000000 00010 01010

X2,  00010,  will enter register file as “Read register 1” as it is Rn  (first register)

X3,  00011,  will enter register file as “Read register 2” as it is Rm (second register)

x10, 01010, will enter the register file as the “Write register” i.e. in which register data should be stored after ALU processing.

SUB X19 ,X9 ,X10

11001011000 01010 000000 01001 10011

X9,   01001,  will enter register file as “Read register 1” as it is Rn  (first register)

X10, 01010,  will enter register file as “Read register 2” as it is Rm (second register)

x19,  10011, will enter the register file as the “Write register” i.e. in which register data should be stored after ALU processing.

ADD X0 ,X19 ,XZR

10001011000 11111 000000 10011 00000

X19,   01011,  will enter register file as “Read register 1” as it is Rn  (first register)

X31(XZR), 11111,  will enter register file as “Read register 2” as it is Rm (second register)

x0,  00000, will enter the register file as the “Write register” i.e. in which register data should be stored after ALU processing.

2)

It is the branch instruction. After executing this instruction, it branches to an address specified by the last 26-bits(which is converted to a 32 bit )

3)

ADD X15, XZR, X10

LSL X15, X15, #15

ORR X11, X15, X11

4. a)

while (i > 0){

i = i-1;

acc=acc+2;

}

b) X0 = 20

c) 5n+2 , instructions will be executed

5.a)

Control Signals:

AluSrc = 0

MemtoReg = 0

MemRead = 0

Memwrite = 0

ALU operation = Hi = 10

RegWrite = Hi = 1

b)

Registers and ALU , both mux (bit 0)

c)

Mux, data memory and sign extend

d)

Data Memory

6)a) 0xF8014062

Sign extend Val = 0xFFFF FFFF F801 4062

Shift Left 2 of 0xF8074062 = 0xE005 0188

After shift left 2 of 0xFFFF FFFF F801 4062 is : 1111111111111111111111111111111111100000000001010000000110001000

=0 xFFFFFFFFE0050188

b)

The bits [9-5] from the instr word and

The the bits [31-0], which is converted to a 64 bit number and that is passed to the ALU

Read Data 1 = ALU input 1 = 00011

Sign extended to 64 bit = ALU input 2 = 0xFFFF FFFF F801 4062

c)

0x22000014

The address from the PC block goes to another adder which adds 4 with it, getting a new address, and then goes to a mux (using bit 0), and that new address is then pass back to the PC block.

d)

mux(before registers)

Taking instr[4-0], 00010 , as input, then the 1 bit in the mux is turned on and then output the instr[4-0] which is 00010

mux(after registers)

Taking in a 64 bit as input , the sign extended-value of instr[31-0], and then the 1 bit is turned on and then output the 64 bit value.

mux(the top one)

Takes the new address as input , the 0 bit is turn on and the output is the new address.

mux(after data memory)

Taks no input and no output

The input in Registers are:

Read register 1 = 00011

Read register 2 = 00010

The output in Registers are :

Read data 1 = 00011

Read data 2 = 00010

e)

i)

ALU input (near data memory):

Input 1 = Read Data 1 = ALU input 1 = 00011

Input 2 = Sign Extend val of inst word [32-0] to 64 bit which is = 0xFFFF FFFF F801 4062

RESULT = 0xFFFF FFFF F801 4065

ii)

Alu adder input (near the shift left 2) = no input

iii)

Adder inputs:  initial address and 4

f)

Instr[4-0] Read register 1 = 00011

Instr[9-5] Read register 2 = 00010

7)

a)

25% + 10 % = 35%

b)

All instructions use the instruction memory. Thus 100%

c)

All uses except R-type instructions. 28% + 25% + 10% +11% 2% = 76%

d)

It is not used/ignored. It does produce an output during each cycle but when not needed it is not used.

8)

a)

R-type = 700ps = 250 + 150+ 200 + 30 +  25 + 25 + 20

b)

Ldur = 950 ps = 250 +250 + 200+ 150 + 25 + 30 + 25 + 20

c)

Stur = 905 ps = 250 + 250 + 200 + 30 + 150 + 25

d)

CBZ  = 950ps = 250 + 250 + 200+ 150 + 25 + 30 + 25 + 20

e)

B  = 705 ps = 250 + 150 +  200 + 25 + 30 + 5 + 25 + 20

f)

I-Type = 700 ps = 30 + 250 + 150 + 25 + 200 + 25 + 20

g)

The control signal given(reg2Loc) to mux, which makes the mux choose which input to give to Register 2

If the value is 1, mux selects instr[4-0] bits from Rt

And if the value is 0, instr[20-16] is selected from the Rm place.